Oil Lines Parts List for Rotec R-2800 Engine

|  | Line From/To | Fitting From (Out) | Fitting To (In) | Line Type /Length | Hose End Out | Hose End In |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Main Oil Pump/Oil Filter* | ⅜” NPT Male † | ⅜” NPT Male † | Braided Steel ~18” | Straight NPT Nut | 90° NPT Nut |
| 2 | Oil Filter/Engine * | ⅜” NPT Male † | ⅜” NPT Male † | Braided Steel | 90° NPT Nut | 90° NPT Nut |
| 3 | Oil Sump (Upper fitting)/Oil Tank | ⅜” NPT Nipple † | AN 840-6D | ⅜” Oil Resistant | ⅜” Clamp | ⅜” Clamp |
| 4 | Tank/ Main Oil Pump | AN840-6D | ⅜” NPT Male † | ⅜” Oil Resistant | ⅜” Clamp | Aeroquip 491-6 |
| 5 | Oil Sump (lower)/ In-line Oil Filter | ⅜” NPT Nipple † | On Filter | ⅜” Oil Resistant | Aeroquip 491-6 | ⅜” Clamp |
| 6 | In-line Oil Filter/Oil Scavenge Pump | On Filter | ⅜” NPT Banjo † | ⅜” Oil Resistant | ⅜” Clamp | ⅜” Clamp |
| 7 | Oil Scavenge Pump/Oil Tank | ⅜” NPT Banjo † | AN840-6D | ⅜” Oil Resistant | ⅜” Clamp | ⅜” Clamp |
| 8 | Sump Drain/⅜”-½” Step-down Fitting‡ | ⅜” NPT Banjo † | AN840-6D | ⅜” Oil Resistant | ⅜” Clamp | ⅜” Clamp |
| 9 | In-line Oil Filter #2 (Optional) ‡ | ‡ | ‡ | ⅜” Oil Resistant | ‡ | ‡ |
| 10 | Step-down Fitting§/Aux Scavenge Pump | AN840-8D | ½” Filter nipple | ½” Oil Resistant | ½” Clamp | ½” Clamp |
| 11 | Aux Oil Scavenge Pump/Oil Tank | ⅜” Filter nipple | AN840-6D | ⅜” Oil Resistant | ⅜” Clamp | ⅜” Clamp |
| 12 | Oil Filter Relief Valve/Oil Tank | ⅜” NPT Male † | ⅜” NPT Male † | ⅜” Oil Resistant | Aeroquip 491-6 | Aeroquip 491-6 |
| 13 | Vent to Air | AN840-6D |  | ⅜” Oil Resistant | ⅜” Clamp |  |

* Will have to have this made up – High pressure hose

† Shipped Installed – Check to be sure

‡ Optional is to install an In-line Oil Filter between Drain Plug Banjo Fitting and the step-Down to protect the Auxiliary Oil Scavenge Pump

§”Step Down Fitting made up of an AN912-4D bushing with an AN840-6D in one end of an AN910-4 Coupling, and an AN840-8D in the other